Colesville Landfill Gas Vent Repairs

On October 24, 2019, work was completed on 11 defective gas vents at the Colesville Landfill by Broome County Landfill staff. Prior to that date all the vents were examined to determine how many were loose and then the necessary materials were ordered. Most vents were not broken but had become loose over time due to defective or aged glue. Vents were removed and the hole was plugged while the area around the pipe was excavated. Plugs were removed and the upper goose-neck piping was glued securely in place. Where appropriate, new couplers were added. Soil was then replaced and packed around the pipes. See various pictures below.