REMEDIAL INVESTIGATION REPORT

for

615 BROADWAY

Kingston, New York

BCP Site No. C356066

Prepared For:

615 Broadway LLC

c/o Baxter Development

11 Garden Street, Suite 101

Poughkeepsie, New York 12601

Prepared By:

Langan Engineering, Environmental, Surveying,

Landscape Architecture and Geology, D.P.C.

368 Ninth Avenue, 8th Floor

New York, New York 10001

December 23, 2025

Project No. 170803001

# TABLE OF CONTENTS

TABLE OF CONTENTS	i

ACRONYM LIST	vi

CERTIFICATION	viii

1.0	INTRODUCTION	1

2.0	SITE PHYSICAL CHARACTERISTICS	3

2.1	Site Description	3

2.2	Surrounding Property Land Use	3

2.3	Site Physical Conditions	4

2.3.1	Topography	4

2.3.2	Regional Geology	4

2.3.3	Regional Hydrogeology	4

2.3.4	Wetlands	5

3.0	SITE BACKGROUND	6

3.1	Historical Operation and Land Use	6

3.2	Proposed Redevelopment Plan	6

3.3	Summary of Previous Environmental Investigations	6

3.4	Summary of Potential Areas of Concern	9

4.0	REMEDIAL INVESTIGATION	11

4.1	Geophysical Survey and Utility Location	12

4.2	Soil Investigation	12

4.2.1	Soil Boring Investigation Methodology	12

4.2.2	Soil Sampling Methodology	13

4.3	Groundwater Investigation	14

4.3.1	Monitoring Well Installation and Development Methodology	14

4.3.2	Groundwater Sampling	15

4.4	Soil Vapor Investigation	15

4.4.1	Soil Vapor Point Installation	16

4.4.2	Soil Vapor Sampling and Analysis	16

4.4.3	Ambient Air Sampling and Analysis	16

4.5	Quality Control Sampling	17

4.5.1	Soil QA/QC Samples	17

4.5.2	Groundwater and Aqueous QA/QC Samples	17

4.5.3	Soil Vapor QA/QC Samples	17

4.6	Data Validation and Reporting	18

4.6.1	Data Usability Summary Report Preparation	18

4.7	Field Equipment Decontamination	19

4.8	Management of Investigative-Derived Waste	19

5.0	Field Observations and Analytical Results	20

5.1	Geophysical Investigation Findings	20

5.2	Geology and Hydrogeology	20

5.2.1	Fill	20

5.2.2	Native Soil Layers	20

5.2.3	Bedrock	20

5.2.4	Hydrogeology	20

5.2.5	Surface Water and Drainage	21

5.3	Soil Findings	21

5.3.1	Field Observations	21

5.3.2	Analytical Results	21

5.4	Groundwater Findings	23

5.4.1	Field Observations	23

5.4.2	Analytical Results	24

5.5	Soil Vapor Findings	25

5.5.1	Analytical Results	25

5.6	Quality Control Results	26

5.7	Data Usability	26

5.8	Evaluation of Potential AOCs	26

5.8.1	AOC 1: Suspected Historic Non-Native Backfilling	26

5.8.2	AOC 2: CVOC Impacts in Groundwater and Soil Vapor	28

5.8.3	AOC 3: Suspect UST in the Northwestern Part of the Site	29

5.8.4	AOC 4: Lead Hotspot in the Northwestern Part of the Site	30

5.8.5	AOC 5: Lead Hotspot in the Western Part of the Site	31

6.0	GREEN REMEDIATION STANDARDS	32

7.0	QUALITATIVE HUMAN HEALTH EXPOSURE ASSESSMENT	34

7.1	Current Conditions	34

7.2	Proposed Conditions	34

7.3	Summary of Environmental Conditions	34

7.4	Conceptual Site Model	35

7.4.1	Potential Sources of Contamination	35

7.4.2	Exposure Media	35

7.4.3	Receptor Populations	36

7.5	Potential Exposure Pathways – On-Site	36

7.5.1	Current Conditions	36

7.5.2	Construction/Remediation Conditions	36

7.5.3	Proposed Future Conditions	37

7.6	Potential Exposure Pathways – Off-Site	37

7.7	Evaluation of Human Health Exposure	38

7.7.1	Current Conditions	38

7.7.2	Construction/Remediation Activities	38

7.7.3	Proposed Future Conditions	39

7.7.4	Human Health Exposure Assessment Conclusions	39

8.0	NATURE AND EXTENT OF CONTAMINATION	41

8.1	Soil Contamination	41

8.2	Groundwater Contamination	41

8.3	Soil Vapor Contamination	41

9.0	CONCLUSIONS	42

10.0	REFERENCES	44

Tables

Table 1	Sample Summary

Table 2	Groundwater Monitoring Well Construction and Elevation Data Summary

Table 3	Soil Sample Analytical Results

Table 4	Lead Delineation Sample Analytical Results

Table 5	Groundwater Sample Analytical Results

Table 6	Soil Vapor and Ambient Air Sample Analytical Results

Figures

Figure 1	Site Location Map

Figure 2	Site Plan

Figure 3	Adjacent Property Land Use Map

Figure 4	Groundwater Contour Map

Figure 5	Sample Location Map and Areas of Concern

Figure 6A	Subsurface Profile A-A’

Figure 6B	Subsurface Profile B-B’

Figure 7A	Soil Sample Analytical Results Map

Figure 7B	Soil Sample Analytical Results Map – Delineation Areas

Figure 8	Groundwater Sample Analytical Results Map

Figure 9	Soil Vapor and Ambient Air Sample Analytical Results Map

Appendices

Appendix A	Proposed Development Plans

Appendix B	Previous Environmental Reports

Appendix C	Site Observation Reports

Appendix D	Photograph Log

Appendix E	Geophysical Survey Report

Appendix F	Soil Boring Logs

Appendix G	Monitoring Well Construction Logs

Appendix H	Groundwater Sampling Logs

Appendix I	Soil Vapor Point Construction and Sampling Logs

Appendix J	Data Usability Summary Reports

Appendix K	Laboratory Analytical Reports

Appendix L	Fish and Wildlife Resources Impact Analysis

# ACRONYM LIST

| Acronym | Definition |
| --- | --- |
| µg/L | micrograms per liter |
| 1,1,1-TCA | 1,1,1-trichloroethane |
| 1,1-DCE | 1,1-dichloroethene |
| 6 NYCRR | Title 6 of the New York Code, Rules, and Regulations |
| AGV | Air Guideline Values |
| AOC | Area of Concern |
| ASTM | American Society for Testing and Materials |
| BCA | Brownfield Cleanup Agreement |
| BCP | Brownfield Cleanup Program |
| bgs | Below Grade Surface |
| CAMP | Community Air Monitoring Plan |
| CEQR | City Environmental Quality Review |
| Cis-1,2-DCE | cis-1,2-dichloroethene |
| COC | Contaminant of Concern |
| CSM | Conceptual Site Model |
| DCP | Department of City Planning |
| DER | Division of Environmental Remediation |
| DNAPL | Dense Non-Aqueous Phase Liquid |
| DUSR | Data Usability Summary Report |
| ELAP | Environmental Laboratory Approval Program |
| ESA | Environmental Site Assessment |
| eV | Electron volt |
| FWRIA | Fish and Wildlife Resources Impact Analysis |
| GPR | Ground Penetrating Radar |
| GV | Guidance Value |
| HASP | Health and Safety Plan |
| IDW | Investigation Derived Waste |
| L/min | Liter per Minute |
| LNAPL | Light Non-Aqueous Phase Liquid |
| mg/kg | Milligram per kilogram |
| µg/m3 | Microgram per cubic meter |
| MS/MSD | Matrix Spike/Matrix Spike Duplicate |
| NAVD88 | North American Vertical Datum of 1988 |
| NYC | New York City |
| NYCDEP | New York City Department of Environmental Protection |
| NYCHA | New York City Public Housing Authority |
| NYCOER | New York City Office of Environmental Remediation |
| NYSDEC | New York State Department of Environmental Conservation |
| NYSDOH | New York State Department of Health |
| PCB | Polychlorinated Biphenyls |
| PCE | Tetrachloroethene |
| PFAS | Per- and polyfluoroalkyl substances |
| PFOA | Perfluorooctanoic acid |
| PFOS | Perfluorooctanesulfonic acid |
| PGW | Protection of Groundwater |
| PID | Photoionization Detector |
| PPE | Personal Protective Equipment |
| ppm | Parts per million |
| PVC | Polyvinyl chloride |
| QA/QC | Quality Assurance/Quality Control |
| RAWP | Remedial Action Work Plan |
| REC | Recognized Environmental Condition |
| RI | Remedial Investigation |
| RIR | Remedial Investigation Report |
| RIWP | Remedial Investigation Work Plan |
| RL | Reporting Limit |
| RURR | 6 NYCRR Part 375 Restricted Use Restricted-Residential |
| SCO | Soil Cleanup Objective |
| SGV | NYSDEC Ambient Water Quality Standards and Guidance Values for Class GA Groundwater |
| SIM | Selected Ion Monitoring |
| SMP | Site Management Plan |
| SPDES | State Pollution Discharge Elimination System |
| SVOC | Semivolatile Organic Compound |
| TAL | Target Analyte List |
| TCL | Target Compound List |
| TOGS | Technical and Operational Guidance Series |
| UN/DOT | United Nations/Department of Transportation |
| USEPA | United States Environmental Protection Agency |
| USGS | United States Geological Survey |
| UST | Underground Storage Tank |
| UU | Unrestricted Use |
| VOC | Volatile Organic Compound |

CERTIFICATION

I, Brian Gochenaur, certify that I am currently a Qualified Environmental Professional as defined in Title 6 of the New York Codes, Rules, and Regulations (NYCRR) Part 375 and that this Remedial Investigation Report (RIR) was prepared in accordance with all applicable statutes and regulations and in substantial conformance with the Division of Environmental Remediation (DER) Technical Guidance for Site Investigation and Remediation (DER-10) and Green Remediation (DER-31).

| DRAFT |  |
| --- | --- |

Brian Gochenaur, QEP

# INTRODUCTION

This Remedial Investigation Report (RIR) was prepared on behalf of 615 Broadway LLC (the Volunteer) for the property located at 615 Broadway (Ulster Country Section 56.109, Tax Block 3, Lot 16) in Kingston, New York (the site). The Volunteer plans to remediate the site in conjunction with a new affordable housing development under the New York State Brownfield Cleanup Program (BCP), pursuant to the Brownfield Cleanup Agreement (BCA), Index No. C356066-11-24, for Site No. C356066 with the New York State Department of Environmental Conservation (NYSDEC), executed on February 26, 2025.

This RIR presents environmental data and findings from the Remedial Investigation (RI), conducted between October 6 and November 26, 2025. The RI was completed by Langan Engineering, Environmental, Surveying, Landscape Architecture and Geology, D.P.C. (Langan) in accordance with Title 6 of the Official Compilation of New York Codes, Rules and Regulations (NYCRR) Part 375-1, 3.8, and 6.8; NYSDEC Division of Environmental Remediation (DER) Technical Guidance for Site Investigation and Remediation (DER-10) and Green Remediation (DER-31); applicable New York State Department of Health (NYSDOH) Guidance for Evaluating Soil Vapor Intrusion in the State of New York (October 2006, with updates May 2017 and February 2024); NYSDEC Guidelines for Sampling and Analysis of Per- and Polyfluoroalkyl Substances (PFAS) Under NYSDEC’s Part 375 Remedial Programs (April 2023); and the NYSDEC-approved Remedial Investigation Work Plan (RIWP) prepared by Langan and dated May 13, 2025. The objectives of the RI included:

Defining the nature and extent of soil, soil vapor, and groundwater contamination at or emanating from the site;

Generating sufficient data to evaluate the remedial action alternatives and prepare a Remedial Action Work Plan (RAWP) to be implemented concurrently with site development; and

Generating sufficient data to evaluate potential threats to human health and the environment from site contamination.

The remainder of this report is organized into the follow sections:

- Section 2.0 describes the site setting and physical characteristics.

- Section 3.0 describes the site background including results of previous investigations and identified areas of concern (AOC).

- Section 4.0 presents the investigation field procedures.

- Section 5.0 describes the field observations and analytical results.

- Section 6.0 describes green remediation principles and techniques implemented in accordance with NYSDEC DER-31.

- Section 7.0 presents an assessment of the exposure risks of site contaminants to human, fish, and wildlife receptors.

- Section 8.0 presents the nature and extent of contamination in all site media as determined through the field investigation and analysis of environmental samples.

- Section 9.0 summarizes the results of the investigation and presents conclusions based on field observations and analytical results.

# SITE PHYSICAL CHARACTERISTICS

## 2.1	Site Description

The about 33,700-square-foot (±0.77-acre) site is located at 615 Boadway in the City of Kingston, New York and is identified on the Ulster County Tax Map as Section 56.109, Tax Block 3, Lot 16. Block 3 is bordered by Henry Street to the north, Broadway to the east, Cedar Street to the south, and Sterling Street to the west. The site is irregularly shaped and includes an about 10-foot-wide by 200-foot-long accessway formerly known as Martin’s Lane that extends to the southeast towards Cedar Street. The site is vacant and improved with an asphalt-paved parking lot and a landscaped area in the eastern part of the site fronting Broadway. A site location map is provided as Figure 1, and a site plan is included as Figure 2.

## 2.2	Surrounding Property Land Use

The site is in an urban setting that is characterized by residential, institutional, and commercial buildings. The following table summarizes surrounding property usage:

| Direction | Adjoining Properties | Surrounding Properties |
| --- | --- | --- |
| North | A gas station (625 Broadway) and three 2-story multi-family residential buildings (16, 20 and 24 Henry Street) followed by Henry Street | Commercial buildings, multi-family residential and apartment buildings, and parking lots |
| East | A 3-story mixed-use commercial and residential building (608 Broadway), a 2-story office building (Taconic Developmental Disabilities Services) and parking lot (609 Broadway), and Broadway followed by a single-story commercial building, multiple 2- to 3-story mixed-use commercial and residential buildings, and a parking lot | Mixed-use commercial and residential buildings |
| South | Salvation Army church and food distribution center (35 Cedar Street) and Cedar Street followed by a 1- to 3-story mixed-use commercial and residential building | Multi-family residential buildings, mixed-use commercial, institutional and residential buildings |
| West | A 2-story multi-family residential building (28 Henry Street) and a 2-story apartment building (55 Cedar Street) | Multi-family residential and apartment buildings |

Public infrastructure (storm drains, sewers, and underground utility lines) exists within the streets surrounding the site. A land use map showing the adjacent and surrounding properties is provided as Figure 3.

Land use within a half mile of the site includes residential, industrial, commercial institutional uses, and parks. The nearest ecological receptor is the Esopus Creek, located about 0.8 miles northwest of the site. Sensitive receptors, as defined in DER-10, located within a half mile of the site include those listed below:

| Name (Approximate distance from Site) | Address |
| --- | --- |
| One Love Learning Daycare (approximately 0.22 miles northeast of the site) | 12 Ardsley Street, Kingston, NY 12401 |
| Lil Sluggers Daycare (approximately 0.22 miles north of the site) | 20 Belvedere Street Kingston, NY 12401 |
| Mi Casita Daycare (approximately 0.46 miles southeast of the site) | 78 W O’Reilly Street Kingston, NY 12401 |
| YWCA Ulster County – Day Care Center (approximately 0.5 miles to the northwest of the site) | 209 Clinton Avenue Kingston, NY 12401 |
| George Washington Elementary School (approximately 0.50 miles southwest of the site) | 67 Wall Street Kingston, NY 12401 |
| Kingston High School (approximately 0.50 miles southeast of the site) | 403 Broadway Kingston, NY 12401 |

## 2.3	Site Physical Conditions

### 2.3.1	Topography

According to survey measurements collected by Langan on October 27, 2025, the elevation at the site ranges from elevations (el) 183.87  and 183.68 in the northeastern and southwestern parts of the site to el 183.08 and 183.07 in the northern part of the site. The site is generally flat with the surrounding area sloping gently towards the north.

### 2.3.2	Regional Geology

The site is located in a developed part of Kingston, New York that is generally covered by paved roads, public walkways, and buildings. The built environment surrounding the site is typically underlain by a layer of uncontrolled (non-native) fill historically emplaced during construction and development of the area since the late 1800s. Based on review of the “Preliminary Geologic Map and Cross Section of the Ellenville and Kerhonkson Quadrangles (In Part) and the Napanoch Quadrangle, Ulster and Orange Counties, New York (Epstein, Jack B., Lyttle, Peter T., dated 1990)”, the site is underlain by bedrock of the Onondaga limestone formation, consisting of medium-gray, cherty fossiliferous limestone.

### 2.3.3	Regional Hydrogeology

Groundwater flow is typically topographically influenced, as shallow groundwater tends to originate in areas of topographic highs and flows toward areas of topographic lows, such as rivers, stream valleys, ponds, and wetlands. A broader, interconnected hydrogeologic network often governs groundwater flow at depth or in the bedrock aquifer. Groundwater depth and flow direction are also subject to hydrogeologic and anthropogenic variables such as precipitation, evaporation, extent of vegetation cover, coverage by impervious surfaces, and subsurface structures. Other factors influencing groundwater include depth to bedrock, the presence of anthropogenic fill, and variability in local geology and groundwater sources or sinks.

Synoptic groundwater level measurements were collected from permanent groundwater monitoring wells on October 22, 2025. Groundwater depth ranged from about 19.53 to 20.25 feet below top of casing, with groundwater elevations ranging from about el 163.73 to el 162.51. Groundwater elevations are highest in the northeastern part of the site, and groundwater flows to the southwest. Groundwater contours are shown on Figure 4.

### 2.3.4	Wetlands

Wetlands were evaluated by reviewing the National Wetlands Inventory and NYSDEC-regulated wetlands map. There are no wetlands located on the site. The closest wetland is a freshwater pond located about 0.38 miles northwest of the site.

# SITE BACKGROUND

This section describes historical site uses and the proposed development and provides a summary of the findings from previous environmental investigations. Potential areas of concern (AOC) were developed in the RIWP based on a review of the previous reports and are summarized in Section 3.4.

## 3.1	Historical Operation and Land Use

Review of historical records and previous environmental reports indicate that the site had been located in a densely-developed urban area characterized by commercial, residential, and industrial uses since the 1890s. The site was historically used for residential and commercial purposes. The site was improved as early as 1887 with a two-story dwelling fronting Broadway and a two-story dwelling in the central part of the site at the end of Martin’s Lane. By 1899, the site was also developed with a one-story store along Broadway. Between 1950 and 1957, the eastern part of the site operated as a produce warehouse (A.H. Guildersleeve & Son) with ancillary buildings for miscellaneous storage, heating supplies, cooling, shipping and receiving, and the western part of the site was developed with multi-story apartments. By 1972, the buildings were demolished and the site was developed with two buildings operated as a motel (Imperial Motel), which remained unchanged through 2006. Records also noted an occupant called “Osgold International (USA)” in 1995; the specific operations of this company are unknown. By 2009, the motel buildings were no longer present. The site contained one building of unknown use in the eastern part of the site alongside a landscaped area between 2013 and 2017, and the remainder of the site consisted of an asphalt-paved parking lot.

## 3.2	Proposed Redevelopment Plan

The proposed development includes the construction of a three- to five-story mixed-use building comprised of two towers which contain residential and commercial units and community space. The ground floor of Tower 1 will contain a lobby, commercial tenant spaces, a bike room, and a mechanical room/office space; the ground floor of Tower 2 will contain an open-air parking lot. The remaining floors will contain residential units. The new building will cover about 21,800 square feet of the site footprint. Areas outside of the building footprint will be developed into a parking lot, paved walkways, landscaped areas, and a plaza. The proposed development plans are included in Appendix A.

## 3.3	Summary of Previous Environmental Investigations

The following environmental reports, previously prepared for the site, were reviewed to inform this RIR and are summarized below. The reports are provided as Appendix B.

- Phase I Environmental Site Assessment (ESA) Report, dated March 2019, prepared by Weston & Sampson

- Phase II ESA Report, dated July 2020, prepared by Weston & Sampson

- Supplemental Subsurface Investigation (SSI Report), dated September 30, 2024, prepared by Langan

Phase I ESA Report, dated March 2019, prepared by Weston & Sampson

Weston & Sampson prepared a Phase I ESA in accordance with American Society for Testing and Materials (ASTM) E-1527-13 standards for the current site owner (City of Kingston). The report identified current and historical uses of surrounding properties as a recognized environmental condition (REC) for the site. The report stated that nearby properties have operated as gasoline service stations, a scrap metal yard, manufacturing, dry cleaning, and a bowling alley, and that inadvertent releases of petroleum and/or hazardous substances associated with these operations may have adversely impacted soil, groundwater or soil vapor at the site.

The Phase I ESA also noted that, due to nearby fuel oil releases and known chlorinated solvent contamination in groundwater at adjoining and surrounding properties, a vapor encroachment condition (VEC) likely exists at the site.

Phase II ESA Report, dated July 2020, prepared by Weston & Sampson

A Phase II ESA was completed by Weston & Sampson in June 2019 to investigate potential on-site contamination. Supplemental groundwater sampling was also conducted in November 2019 at the site. Findings from the Phase II ESA are summarized below:

Subsurface Observations

The site is underlain by fill consisting of dark brown fine- to medium-grained sand with varying amounts of gravel, concrete, clinkers, brick and slag to between about 1 to 5 feet bgs followed by tan fine- to coarse and fine- to medium-grained sands with varying amounts of gravel to about 20 feet bgs. Tan clay lenses were observed between 22 and 25 feet bgs in the northern part of the site and between 16 and 19 feet bgs in the southern part of the site. Odors and staining were not observed in soil during the Phase II ESA. A maximum photoionization detector (PID) reading of 19.5 parts per million (ppm) was observed in one soil boring (SB-2) between 0 and 1 feet bgs in the northern part of site. Bedrock was not encountered.

Groundwater was encountered at about 20 feet bgs in monitoring wells installed across the site. Based on the investigation findings, groundwater was observed to flow to the south-southwest.

Analytical Results

- Soil: Semivolatile organic compounds (SVOC), particularly polycyclic aromatic hydrocarbons (PAH), (benzo(a)anthracene, benzo(a)pyrene, benzo(b)fluoranthene, chrysene, indeno(1,2,3-cd)pyrene), and metals (lead) were detected in the fill layer at concentrations exceeding the NYCRR Part 375 Unrestricted Use (UU), Protection of Groundwater (PGW), and/or Restricted Use Restricted-Residential (RURR) soil cleanup objectives (SCO).

- Groundwater: Chlorinate volatile organic compounds (CVOC) (tetrachloroethene [PCE] and trichloroethene [TCE]), and total metals (selenium) were detected in groundwater at concentrations above the NYSDEC Division of Water Technical and Operational Guidance Series (TOGS) 1.1.1 Ambient Water Quality Standards (AWQS) and Guidance Values for Drinking Water (class GA) (collectively referred to as the “SGVs”). Based on the analytical results of CVOCs in groundwater, a spill was reported to the NYSDEC on July 15, 2019 and Spill No. 1903819 was assigned to the site.

- Soil Vapor: Soil vapor samples were not collected as part of the Phase II ESA.

Spill No. 1903819 was administratively closed by the NYSDEC in 2020 because CVOC impacts were previously discovered in groundwater in the vicinity of the site; however, an interview between Langan and Weston & Sampson on November 30, 2023 confirmed that no remediation had occurred at the site.

SSI Report, dated September 30, 2024, prepared by Langan

Langan completed an SSI on August 12, 2024 to further identify soil contamination attributed to historical site operations and confirm that a VEC exists at the site due to known chlorinated solvent contamination in groundwater. Findings from the SSI are summarized below:

Geophysical Survey

The survey identified sewer, electric, and unspecified utilities, as well as sewer mains and an electric box within the site. In addition, a geophysical anomaly was identified in the northwestern part of the site which may indicate the presence of an underground storage tank (UST).

Subsurface Observations

The site is underlain by fill, predominantly consisting of brown to tannish-brown fine-grained sand with varying amounts of fine gravel, silt, brick, glass, and slag which extends to between about 1 to 3 feet bgs. Tannish-brown to dark-brown fine sand with varying amounts of fine gravel and silt was observed below the fill layer. A maximum PID reading of 1.9 ppm was observed in soil boring SB07 at about 1.75 feet bgs. Odors and staining were not observed. Groundwater was observed at about 19 feet bgs in soil borings advanced to 20 feet bgs across the site (SB01, SB07, and SB09).

Analytical Results

- Soil: SVOCs, specifically PAHs, and metals were detected in soil samples across the site at concentrations exceeding the RURR SCOs. SVOCs including benzo(a)anthracene, benzo(a)pyrene, benzo(b)fluoranthene, and indeno(1,2,3-cd)pyrene were detected above RURR SCOs in soil borings SB07 and SB10. One additional SVOC, dibenz(a,h)anthracene, was also identified above the RURR SCO in soil boring SB10. Lead was detected above the RURR SCO in soil borings SB07 and SB09. Mercury was also detected above the RURR SCO in soil boring SB07.

- Groundwater: Groundwater samples were not collected as part of the SSI.

- Soil Vapor: Total volatile organic compound (VOC) concentrations detected in soil vapor samples ranged from 1,185.678 micrograms per cubic meter (µg/m3) in SV03 to 1,596.302 µg/m3 in SV01. Maximum TCE and PCE concentrations of 176 µg/m3 and 228 µg/m3 were detected in SV03 and SV01, respectively. Total benzene, toluene, ethylbenzene, and xylenes (BTEX) concentrations ranged from 219.32 µg/m3 in SV01 to 275.19 µg/m3 in SV02.

## 3.4	Summary of Potential Areas of Concern

Based on site observations, development history, and the findings of previous environmental investigations, the potential AOCs identified in the RIWP and investigated during this RI are described below. These AOCs are also shown on Figure 5.

Potential AOC 1: Suspected Historic Non-Native Backfilling

Non-native fill material, predominantly consisting of brown to tannish-brown fine-grained sand with varying amounts of fine gravel, silt, brick, glass, and slag extends from about 1 to 5 feet bgs across the site. The fill was likely placed at the site as part of grading prior to historic construction and may contain the remnants of former building materials. Contaminants associated with non-native fill were identified in shallow soil samples (0 to 2 feet bgs) during the Phase II ESA and SSI, including concentrations of five SVOCs and two metals, exceeding the UU, PGW, and/or RURR SCOs.

Potential AOC 2: CVOC Impacts in Groundwater and Soil Vapor

The Phase II ESA and SSI identified the presence of TCE and PCE in groundwater and soil vapor across the site. During the Phase II ESI, concentrations of TCE and/or PCE exceeded the SGVs in three groundwater monitoring wells across the site. Maximum TCE and PCE concentrations of 176 µg/m3 and 228 µg/m3 were detected during the SSI in soil vapor samples SV03 and SV01, respectively, which confirm a VEC exists at the site.

Potential AOC 3: Suspect UST in the Northwestern Part of the Site

During the SSI, a geophysical anomaly was identified in the northwestern part of the site which may indicate the presence of a UST. Additional borings will be advanced in the vicinity of the UST to further investigate the potential for petroleum-related impacts to the subsurface.

Potential AOC 4: Lead Hotspot in the Northwestern Part of the Site

During the Phase II ESA, lead was detected at a concentration of 3,000 milligram per kilogram (mg/kg) in the northwestern part of the site from a surficial soil sample (0 to 2 feet bgs). The extent of elevated lead concentrations has not been horizontally nor vertically delineated in this area.

Potential AOC 5: Lead Hotspot in the Western Part of the Site

During the SSI, lead was detected at a concentration of 1,040 mg/kg from 1 to 2 feet bgs in the western part of the site. The extent of elevated lead concentrations has not been horizontally nor vertically delineated in this area.

# REMEDIAL INVESTIGATION

The RI was completed between October 6 and November 26, 2025 to investigate potential AOCs and to determine, to the extent practical, the nature and extent of contamination in soil, groundwater, and soil vapor. The scope of the RI included the field tasks listed below to supplement the data and findings of previous investigations. A summary of samples collected, with a rationale for each investigation point in relation to the AOCs, is provided in Table 1. Sample locations and AOCs are presented on Figure 5.

The Project Manager or Field Team Leader submitted daily site observation reports to the NYSDEC Project Manager during the RI. Site observation reports are included in Appendix C. A digital photograph log documenting activities accomplished during the RI is included in Appendix D.

The RI consisted of the following:

Geophysical Survey

Completion of a geophysical survey to clear sample locations and identify anomalies indicative of subsurface utilities and structures, including the suspect UST in the northwestern part of the site, in the vicinity of each boring location.

Soil Boring Advancement and Sampling

Advancement of 40 soil borings to a maximum depth of 25 feet bgs.

Continuous field screening of recovered soil for environmental impacts using visual and olfactory methods and with a PID equipped with a 10.6 electron volt (eV) lamp.

Collection of 81 grab soil samples (plus quality assurance/quality control [QA/QC] samples) for laboratory analysis.

Monitoring Well Installation, Sampling, and Gauging

Installation and development of nine permanent monitoring wells to a maximum depth of 25 feet bgs.

Collection of one groundwater sample from each of the newly installed permanent monitoring wells, for a total of nine groundwater samples (plus QA/QC samples) for laboratory analysis.

Surveying and gauging of existing and newly installed monitoring wells to evaluate groundwater elevation.

Development of a groundwater contour map.

Soil Vapor Point Installation and Soil Vapor/Ambient Air Sampling

Installation of ten soil vapor points to a depth of about 5 feet bgs.

Collection of one soil vapor sample from each point and one outdoor ambient air sample for laboratory analysis.

The RI was conducted in accordance with DER-10, the NYSDOH Guidance for Evaluating Soil Vapor Intrusion in the State of New York (NYSDOH Guidance), and the NYSDEC-approved RIWP.

## 4.1	Geophysical Survey and Utility Location

Prior to commencement of ground-intrusive RI field activities (i.e., soil borings, well installation, and soil vapor point installation), NOVA Geophysical Services (NOVA) of Douglaston, New York, completed a geophysical survey on October 6, 2025 under the supervision of Langan field personnel. NOVA used ground-penetrating radar (GPR) and electromagnetic detection equipment to identify potential USTs, buried utilities, and subsurface anomalies across the site and at proposed sample locations. Borings were relocated as necessary to avoid subsurface conflicts. A copy of the geophysical survey report is included as Appendix E.

## 4.2	Soil Investigation

### 4.2.1	Soil Boring Investigation Methodology

A total of 40 soil borings were advanced during the RI by Lakewood Environmental Services, Corp. (Lakewood) of Smithtown, New York. Boring locations were selected to evaluate potential AOCs listed in Section 3.4 and to supplement previous environmental investigations. Soil borings were advanced using a Geoprobe® 6610DT track-mounted drill rig. Eight borings (RISB01 through RISB07, RISB09 through RISB13, and RISB22) were pre-cleared with a hand auger to a depth of 5 feet bgs prior to advancement with the drill rig. Soil borings were advanced to between 4 and 25 feet bgs across the site. A map showing the boring locations is presented as Figure 5. The following table indicates the soil borings associated with their respective potential AOC and advancement depths:

| Potential AOC | Associated Soil Borings | Advancement Depth |
| --- | --- | --- |
| AOC 1 – Suspected Historic Non-Native Backfilling | RISB01 to RISB23 | 10 to 25 feet bgs |
| AOC 2 – CVOC Impacts in Soil and Groundwater |  |  |
| AOC 3 – Suspect UST in the Northwestern Part of the Site | RISB24 to RISB26 | 10 feet bgs |
| AOC 4 – Lead Hotspot in the Northwestern Part of the Site | RISB18, RISB18_NE1, RISB18_NE2, RISB18_SE1, RISB18_SE2, RISB18_SW1, and RISB18_SW2 | 4 feet bgs |
| AOC 5 – Lead Hotspot in the Western Part of the Site | RISB21, RISB21_N1, RISB21_N2, RISB21_E1, RISB21_E2, RISB21_S1, RISB21_S2, RISB21_W1, and RISB21_W2 | 4 feet bgs |

Soil samples were collected into 4-foot-long acetate liners using a 2-inch-diameter Macrocore® sampler using direct-push technology. Discrete soil samples were collected from the surface to the final depth of each boring and were screened for visual, olfactory, and instrumental evidence of environmental impacts and visually classified for soil type, grain size, texture, and moisture content. Instrument screening for the presence of VOCs was performed with a PID equipped with a 10.6-eV lamp. Langan personnel documented the work, logged the soil type, screened the soil samples for environmental impacts, and collected environmental samples for laboratory analyses. Soil boring logs are presented in Appendix F.

### 4.2.2	Soil Sampling Methodology

During implementation of the RI, 81 grab soil samples (plus QA/QC samples) were collected for laboratory analysis to investigate the potential AOCs and provide vertical and horizontal delineation of contaminants. The sampling methodology for each potential AOC is described below:

- Three grab soil samples were collected from each of the boring locations RISB01 through RISB23 to investigate potential AOCs 1 and 2. The first sample was collected from the 0- to 2-foot bgs interval. A second sample was collected from bottom of the fill layer, and a third sample was collected from the first underlying depth interval without evidence of fill (i.e., native soil).

- One grab soil sample was collected from each of the boring locations RISB24 through RISB26 at the interval beneath the suspected UST bottom (i.e., about 8 to 10 feet bgs) to investigate potential AOC 3.

- Lead delineation borings were advanced from five- and ten-foot step-outs to the north, south, east, and west from boring RISB21 and from five- and ten-foot step-out borings to the northeast, southeast, and southwest from boring RISB18 to investigate potential AOCs 4 and 5. Two samples were collected from each of the fourteen borings: one from the impacted depth interval (0 to 2 feet bgs) and one from the two-foot interval below the known impacts.

Samples submitted for VOC analysis were collected directly from the acetate liner or from hand-augured soil via laboratory-supplied Terra Core® soil samplers. The remaining sample volume was homogenized and placed in appropriate laboratory-supplied containers. A dedicated pair of nitrile gloves were donned to collect each per- and polyfluoroalkyl substances (PFAS) sample to limit cross-contamination.

The sample containers were labeled, placed in laboratory-supplied coolers and packed on ice (to maintain a temperature of temperature of approximately 4 degrees Celsius). The sample coolers were picked up and delivered via courier under standard chain-of-custody protocol to Pace Analytical Services, LLC (Pace), a NYSDOH Environmental Laboratory Approval Program (ELAP)-certified analytical laboratory (ELAP ID No. 11148) in Westborough, Massachusetts.

Soil samples from soil borings RISB01 through RBSB23 were analyzed for Part 375/Target Compound List (TCL) VOCs and SVOCs, polychlorinated biphenyls (PCB), pesticides, herbicides, Total Analyte List (TAL) metals including hexavalent and trivalent chromium, total cyanide, 1,4-dioxane and PFAS. Soil samples collected from soil borings RISB24 through RISB26 were analyzed for Part 375/TCL VOCs and SVOCs. Soil samples collected from lead delineation soil borings were analyzed for total lead and, when warranted, for Toxicity Characteristic Leaching Procedure (TCLP) lead. A sample summary is provided as Table 1.

Following sample collection, borings which were not converted to groundwater monitoring wells were backfilled with bentonite and clean soil cuttings without evidence of environmental impacts; the surface of each boring was sealed with an asphalt or concrete patch.

## 4.3	Groundwater Investigation

Langan field personnel documented conversion of 9 soil borings (RISB01, RISB02, RISB05, RISB10, RISB13, RISB15, RISB17, RISB18, and RISB22) into permanent groundwater monitoring wells (RIMW01, RIMW02, RIMW05, RIMW10, RIMW13, RIMW15, RIMW17, RIMW18, and RIMW22) by Lakewood. One groundwater sample was collected from each of the monitoring wells to characterize groundwater conditions and to investigate potential groundwater impacts associated with the potential AOCs. Additional groundwater samples were collected for QA/QC purposes. Monitoring well locations are presented on Figure 5.

### 4.3.1	Monitoring Well Installation and Development Methodology

Permanent monitoring wells were constructed with 2-inch-diameter, threaded, flush-joint, polyvinyl chloride (PVC) casing and 0.01-inch slotted pre-packed well screens (10 feet in length). Monitoring wells were constructed so that the well screen straddled the observed groundwater table. Clean sand was used to backfill the 2-inch annulus around the riser to about 2 feet above the top of the pre-packed well screen. A 2-foot-thick bentonite seal was installed above the sand, and the remaining borehole annulus was backfilled with drill cuttings exhibiting no evidence of petroleum impacts to within 12 inches of the surface and/or grouted to the surface. Monitoring wells were finished with flush-mount metal manhole covers encased in concrete.

Following installation, the permanent monitoring wells were surged and developed with a submersible pump until the water became clear (having turbidity less than 50 Nephelometric Turbidity Units [NTU]). Purged groundwater was stored in a labeled 55-gallon New York State Department of Transportation (DOT)-approved drum staged on-site pending off-site disposal. Monitoring well construction details are included in Table 2, and construction logs are provided in Appendix G.

Langan field personnel completed synoptic groundwater gauging on October 22, 2025 using a Solinst oil/water interface probe. The monitoring well top-of-casing elevations were surveyed by Langan on October 27, 2025. Groundwater elevations ranged from el 163.73 in RIMW02 to el 162.51 in RIMW17. Groundwater elevations are presented in Table 2. A groundwater elevation contour map is presented as Figure 4.

### 4.3.2	Groundwater Sampling

One groundwater sample was collected from each permanent monitoring well. Samples were collected in accordance with the procedures set forth in the United States Environmental Protection (USEPA) guidance (“Low Stress [low flow] Purging and Sampling Procedure for the Collection of Groundwater Samples from Monitoring Wells”, dated July 30, 1996 and revised September 19, 2017) to allow for collection of a representative sample. Monitoring wells were purged and physical and chemical parameters (e.g., temperature, dissolved oxygen, oxygen reduction potential, and turbidity) were allowed to stabilize to ranges specified in the USEPA guidance before they were sampled. Because samples were collected for PFAS compounds, monitoring wells were not gauged for static water levels or drawdown during purging to limit cross-contamination. Wells were sampled using a peristaltic pump with dedicated high-density polyethylene (HDPE) tubing. Purge water was containerized into labeled 55-gallon DOT-approved drums for off-site disposal. Groundwater sampling logs are included in Appendix H.

Nine groundwater samples (one sample from each well) were collected into laboratory-supplied glassware, packed with ice to maintain a temperature of approximately 4 degrees Celsius, and transported via courier service to Pace under chain-of-custody protocols. Groundwater samples collected from the permanent groundwater monitoring wells were analyzed for Part 375/TCL VOCs, SVOCs, PCBs, pesticides, herbicides, TAL total and dissolved metals, trivalent and hexavalent chromium, total cyanide, 1,4-dioxane and PFAS. A dedicated pair of nitrile gloves were donned to collect each PFAS sample in order to limit cross-contamination.

## 4.4	Soil Vapor Investigation

DER-10 requires an assessment of soil vapor for contaminated sites to evaluate the potential exposure to VOCs through vapor intrusion into occupied spaces. Ten soil vapor points (RISV01, RISV02, RISV05, RISV08, RISV10, RISV13, RISV15, RISV17, RISV18, and RISV22) were installed by Lakewood to identify potential impacts associated with historical site use and evaluate the potential AOC identified in the RIWP. One ambient air sample (AA01) was collected for QA/QC purposes. Soil vapor sample locations are presented on Figure 5.

### 4.4.1	Soil Vapor Point Installation

Soil vapor points were installed by Lakewood using a Geoprobe® 6610DT drill rig and were advanced to 5 feet bgs. A polyethylene vapor implant (approximately 2 inches in length) was threaded to Teflon-lined, 1/4-inch-diameter polyethylene tubing and lowered to the bottom of the hole in accordance with the NYSDOH Guidance. A sand filter pack was installed around the screen implant by pouring No. 2 sand into the annulus to about 12 inches bgs. The remainder of the annulus was filled to grade surface with a hydrated bentonite seal. Soil vapor construction logs are included in Appendix I.

### 4.4.2	Soil Vapor Sampling and Analysis

As a QA/QC measure, an inert tracer gas (helium) was introduced into an above-grade sampling chamber to ensure that the soil vapor points were properly sealed above the target sampling depth, thereby preventing subsurface infiltration of ambient air. Direct readings of less than 10 percent helium in the sampling tube were considered sufficient to verify a tight seal at each soil vapor point.

Each soil vapor point was purged using a MultiRAE meter at a rate less than 0.2 liters per minute (L/min) to evacuate a minimum of three sample tubing volumes prior to sample collection. The purged soil vapor was also monitored for VOCs and the values were recorded. After purging was completed, the soil vapor samples were collected into laboratory-supplied, batch-certified, 6-Liter Summa® canisters calibrated for 2 hours of sampling. Soil vapor sampling logs are included in Appendix I.

Summa® canisters were labeled and transported via courier to Pace under standard chain-of-custody protocol. Soil vapor air samples were analyzed for VOCs by USEPA Method TO-15.

### 4.4.3	Ambient Air Sampling and Analysis

One outdoor ambient air sample was collected concurrently with the soil vapor samples. The ambient air sample was collected from the central part of the site about 5 feet above the ground surface (i.e., breathing height) to evaluate external influences on soil vapor quality.

Outdoor ambient air sampling was conducted in general accordance with the NYSDOH Guidance. Prior to sample collection, the area was screened using a MultiRAE meter to identify potential sources of organic vapors that may interfere with sampling. The ambient air sample was collected into a laboratory-supplied, batch-certified, 6-liter Summa® canister. Outdoor ambient air sample AA01 was calibrated for 2 hours of sampling, concurrent with soil vapor sample collection.

The Summa® canisters were labeled and transported via courier to Pace under standard chain-of-custody protocols. The ambient air sample was analyzed for VOCs by USEPA Method TO-15.

## 4.5	Quality Control Sampling

Field blanks, PFAS field blanks, trip blanks, field duplicate samples, and matrix spike/matrix spike duplicate (MS/MSD) samples were collected and submitted for laboratory analysis for QA/QC purposes. QA/QC samples are detailed in Table 1 and are summarized below:

### 4.5.1	Soil QA/QC Samples

- Six field duplicate samples

- Six MS/MSD samples

- Six field blanks

- Three trip blanks

### 4.5.2	Groundwater and Aqueous QA/QC Samples

- One field duplicate sample

- One MS/MSD sample

- One field blank

- Three PFAS field blanks

- Three trip blanks

### 4.5.3	Soil Vapor QA/QC Samples

- One ambient air sample

Field duplicates were collected to assess the precision of the analytical methods relative to the sample matrix. The duplicates were collected from the same material as the primary sample by splitting the volume of homogenized sample collected in the field into two sample containers.

MS/MSD samples were collected to assess the effect of the sample matrix on the recovery of target compounds or target analytes. MS/MSD samples were collected from the same material as the primary sample by splitting the volume of the homogenized sample collected in the field into three sample containers.

The trip blank samples were collected to assess the potential for contamination of the sample containers and samples during the trip from the laboratory, to the field, and back to the laboratory for analysis. Trip blanks contain about 40 milliliters of acidic water (doped with hydrochloric acid) that is sealed by the laboratory when the empty sample containers are shipped to the field, and unsealed and analyzed by the laboratory when the sample shipment is received from the field. The trip blank samples were analyzed for VOCs.

## 4.6	Data Validation and Reporting

Analytical data was submitted to a Langan validator in accordance with USEPA and NYSDEC validation protocols. Copies of the data usability summary reports (DUSR) are provided in Appendix J.

### 4.6.1	Data Usability Summary Report Preparation

A DUSR was prepared for each sampling matrix following data validation. The DUSR presents the results of data validation, including a summary assessment of laboratory data packages, sample preservation and chain-of-custody procedures, and a summary assessment of precision, accuracy, representativeness, comparability, and completeness for each analytical method.

For each of the organic analytical methods, the following was assessed:

- Holding times

- Instrument tuning

- Instrument calibrations

- Blank results

- System monitoring compounds or surrogate recovery compounds (as applicable)

- Internal standard recovery results

- MS/MSD results

- Target compound identification

- Chromatogram quality

- Compound quantization and reported detection limits

- System performance

- Results verification

For each of the inorganic compounds, the following was assessed:

- Holding times

- Calibrations

- Blank results

- Interference check sample

- Laboratory check samples

- Duplicates

- Matrix Spike

- Inductively Coupled Plasma Atomic Emission Spectrometry (ICP-AES) QC

- ICP serial dilutions

- Results verification and reported detection limits

Based on the results of data validation, the following qualifiers may be assigned to the data in accordance with USEPA’s guidelines and best professional judgment:

“U” – The analyte was analyzed for but was not detected at a level greater than or equal to the reporting limit (RL) or the sample concentration or the sample concentration for results impacted by blank contamination.

“UJ” – The analyte was not detected at a level greater than or equal to the RL; however, the reported RL is approximate and may be inaccurate or imprecise.

“J” – The analyte was positively identified, and the associated numerical value is the approximate concentration of the analyte in the sample.

After data validation was complete, validated data was used to prepare the tables and figures included in this report. Analytical results will be provided in the NYSDEC’s electronic data deliverable (EDD) format for EQuIS™.

## 4.7	Field Equipment Decontamination

A peristaltic pump with dedicated HDPE tubing was used to sample each groundwater monitoring well. The groundwater sampling equipment, including a water quality meter, were cleaned with Alconox and rinsed with deionized water between sampling locations during groundwater sample collection. Decontamination occurred at the sampling locations and all liquids were temporarily contained in a 5-gallon bucket. Decontamination wastewater was containerized in 55-gallon DOT-approved drums for off-site disposal.

## 4.8	Management of Investigative-Derived Waste

Decontamination and well development/purging fluids (investigation-derived wastes [IDW]) were containerized in two 55-gallon DOT-approved drums with closed tops. The drums were properly labeled, sealed, and staged on-site during the RI. On November 26, 2025, Lakewood exported the drums from the site to the Clean Water of New York facility located in Staten Island, New York for off-site disposal.

# Field Observations and Analytical Results

## 5.1	Geophysical Investigation Findings

The geophysical survey performed during the RI identified an inactive electric line, a drainage line with two drains, and linear anomalies running from the central part of the site to the east toward Broadway. In addition, one anomaly and one area of high noise (indicating the potential presence of debris or disturbance) were identified in the northwestern and southwestern parts of the site. UST appurtenances, fill ports and/or fill lines were not identified. No anomalies consistent with USTs were identified in the surveyed areas of the site. During the survey, boring locations were relocated as necessary to avoid subsurface conflicts. The survey was generally consistent with the previous survey conducted by NOVA during the SSI. A copy of the geophysical report is included in Appendix E of this RIR.

## 5.2	Geology and Hydrogeology

This section describes the geological and hydrogeological observations made during the RI. A groundwater contour map is provided as Figure 4 and cross-sectional diagrams showing inferred soil profiles are shown on Figure 6A and 6B. Soil boring logs are provided in Appendix F.

### 5.2.1	Fill

The site is underlain by a fill layer that extends from below the surface cover to depths ranging from about 1 to 5 feet bgs. The bottom of the fill layer was observed to be most shallow in the northwestern part of the site (RISB18_NE1, RISB18_SE1, RISB18_SE2, RISB19, RISB24, RISB25, and RISB26) and deepest in the northeastern part of the site (RISB03, RISB04, and RISB06). The fill predominantly consists of tannish brown to dark brown fine-grained sand with varying amounts of silt, clay, gravel, brick, concrete, glass, woody vegetation, and coal.

### 5.2.2	Native Soil Layers

The fill layer is underlain by native soil that predominantly consists of tan to orangish brown fine-grained sand with varying amounts of clay, silt, and gravel, which extended to the termination depth of each boring. Intermittent layers of orange-brown clay were observed throughout soil borings in the western part the site from depths ranging from 1 to 6 feet bgs.

### 5.2.3	Bedrock

Bedrock was not encountered during the RI or during previous investigations.

### 5.2.4	Hydrogeology

Synoptic groundwater level measurements were collected from permanent groundwater monitoring wells installed as part of the RI on October 22, 2025. Groundwater depth ranged from about 19.53 to 20.25 feet below top of casing, with groundwater elevations ranging from about el 163.73 to el 162.51. Groundwater elevation is highest in the northeastern part of the site, and groundwater flow at the site generally flows to the southwest. Groundwater elevations are summarized in Table 2 and Appendix G, and a groundwater contour map is presented as Figure 4.

### 5.2.5	Surface Water and Drainage

The surface of the site is primarily comprised of an asphalt-paved parking lot. Surface water does not exist at the site.

## 5.3	Soil Findings

### 5.3.1	Field Observations

Field evidence of petroleum or other hazardous substance impacts was not observed in the RI soil borings. PID readings were not detected above background concentrations in any soil boring, indicating no detectable levels of VOCs present.

### 5.3.2	Analytical Results

A total of 69 soil samples, plus four field duplicates, were collected and analyzed for Part 375/TCL VOCs and SVOCs, PCBs, pesticides, herbicides, Part 375/TAL metals including hexavalent and trivalent chromium, total cyanide, 1,4-dioxane and PFAS. Nine soil samples, plus one field duplicate, were collected analyzed for TCLP and/or total lead. Three samples, plus one field duplicate, were collected and analyzed for Part 375/TCL VOCs and SVOCs.

RI soil sample analytical results for RISB01 through RISB26 are summarized in Table 3 with comparisons to UU, RURR, and Protection of Groundwater (PGW) SCOs, and the NYSDEC April 2023 PFAS guidance values (PFAS guidance values). Soil samples collected from RISB18, RIBSB21, and their respective lead delineation borings, are summarized in Table 4. Soil sample analytical results are shown on Figure 7A. Delineation sample analytical results are shown on Figure 7B. Laboratory analytical reports are included in Appendix K.

The following contaminants were detected at concentrations exceeding UU SCOs. Exceedances of the RURR SCO are bolded in the following in-text summary tables.

VOCs, PCBs, Herbicides

VOCs, total PCBs, and herbicides were not detected at concentrations above the UU and/or RURR SCOs.

SVOCs

Six SVOCs were detected at concentrations above the UU and/or RURR SCOs in five soil borings (RISB01, RISB06, RISB11, RISB15, and RISB22) at depths ranging from surface grade to 5 feet bgs. The following table summarizes SVOC exceedances:

| Analyte | Minimum Detected Concentration above SCO | Maximum Detected Concentration above SCO | SCOs | Frequency of Detection above UU and/or RURR SCO |
| --- | --- | --- | --- | --- |
| Benzo(a)anthracene | 1.1 mg/kg in RISB11_2-4 | 1.9 mg/kg in RISB22_3-5 | UU:1 mg/kg RURR: 1 mg/kg | 4 |
| Benzo(a)pyrene | 1.4 mg/kg in RISB11_0-2 and RISB15_0-2 | 2.1 mg/kg in RISB22_3-5 | UU:1 mg/kg RURR: 1 mg/kg | 3 |
| Benzo(b)fluoranthene | 1.2 mg/kg in RISB06_3-5 and RISB11_2-4 | 2.7 mg/kg in RISB22_3-5 | UU:1 mg/kg RURR: 1 mg/kg | 5 |
| Chrysene | 1.4 mg/kg in RISB15_0-2 | 1.9 mg/kg in RISB22_3-5 | UU:1 mg/kg | 3 |
| Dibenz(a,h)anthracene | 0.41 mg/kg in RISB22_3-5 |  | UU:0.33 mg/kg RURR: 0.33 mg/kg | 1 |
| Indeno(1,2,3-cd)pyrene | 0.54 mg/kg in RISB01_0-2 | 1.6 mg/kg in RISB22_3-5 | UU:0.5 mg/kg RURR: 0.5 mg/kg | 6 |

Pesticides

Four pesticides were detected at concentrations above UU SCOs in eight soil borings (RISB01 through RISB03, RISB09, RISB14, RISB20, and RISB22) at depths ranging from surface grade to 6 feet bgs. The following table summarizes the pesticides exceedances:

| Analyte | Minimum Detected Concentration above SCO | Maximum Detected Concentration above SCO | SCOs | Frequency of Detection above UU SCO |
| --- | --- | --- | --- | --- |
| 4,4'-DDE | 0.00764 mg/kg in RISB14_1-2 |  | UU:0.0033 mg/kg | 1 |
| 4,4'-DDT | 0.00418 mg/kg in SODUP01_100725 | 0.0154 mg/kg in RISB14_1-2 | UU:0.0033 mg/kg | 5 |
| Alpha Chlordane | 0.0963 mg/kg in RISB09_2-4 | 2.6 mg/kg in RISB01_0-2 | UU:0.94/kg | 11 |
| Heptachlor | 0.0448 mg/kg in RISB02_2-4 | 0.717 mg/kg in RISB01_0-2 | UU:0.042 mg/kg | 6 |

- Soil sample RISB06_0-2 is the parent sample of SODUP01_100725.

Metals

Five metals were detected at concentrations above the UU and/or RURR SCOs in 20 soil borings (RISB03 through RISB07, RISB09 through RISB16, RISB18, RISB18_NE1, RISB18_SW1,and RISB19 through RISB22) at depths ranging from surface grade to 5 feet bgs. The following table summarized the metals exceedances:

| Analyte | Minimum Detected Concentration above SCO | Maximum Detected Concentration above SCO | SCOs | Frequency of Detection above UU and/or RURR SCO |
| --- | --- | --- | --- | --- |
| Copper | 55.2 mg/kg in RISB15_0-2 |  | UU:50 mg/kg | 1 |
| Lead | 63.1 mg/kg in RISB14_2-4 | 1,040 mg/kg in RISB19_0-1 | UU:63 mg/kg RURR: 400 mg/kg | 30 |
| Mercury | 0.185 mg/kg in RISB11_0-2 | 1.39 mg/kg in RISB15_0-2 | UU:0.18 mg/kg RURR: 0.81 mg/kg | 16 |
| Nickel | 33 mg/kg in RISB07_3-5 | 152 mg/kg in RISB22_3-5 | UU:30 mg/kg | 4 |
| Zinc | 111 mg/kg in RISB03_3-5 | 297 mg/kg in RISB20_0-2 | UU:109 mg/kg | 6 |

PFAS

Perfluorooctanoic acid (PFOA) and perfluorooctanesulfonic acid (PFOS) concentrations in soil were compared to the guidance values established in the April 2023 Guidelines for Sampling, Analysis and Assessment of PFAS. PFOS was detected in eight soil samples from six soil borings (RISB01, RISB03, RISB06, RISB09, RISB11, and RISB23) above the UU SCO established in the NYSDEC PFAS Guidance at depths ranging from surface grade to 5 feet bgs. PFOS concentrations above the UU SCO ranged from 0.000933 mg/kg in RISB03_0-2 to 0.00272 mg/kg in RISB09_2-4.

Protection of Groundwater Evaluation

Compounds detected in soil during the RI, which were also present in groundwater at concentrations above the SGVs, were also compared to the PGW SCOs and guidance values for PFAS. Table 3 includes soil analytical result comparisons to the PGW SCOs and guidance values for PFAS. The following table summarizes the applicable compounds:

| Analyte | Minimum Detected Concentration above SCO | Maximum Detected Concentration above SCO | SCOs | Frequency of Detection above PGW SCO |
| --- | --- | --- | --- | --- |
| SVOCs |  |  |  |  |
| Benzo(a)anthracene | 1.1 mg/kg in RISB11_2-4 | 1.9 mg/kg in RISB22_3-5 | PGW: 1 mg/kg | 4 |
| Pesticides |  |  |  |  |
| Heptachlor | 0.717 mg/kg in RISB01_0-2 |  | PGW: 0.38 mg/kg | 1 |
| PFAS |  |  |  |  |
| Perfluorooctanesulfonic Acid (PFOS) | 0.00108 mg/kg in RISB01_0-2 | 0.00272 mg/kg in RISB09_2-4 | PGW: 0.001 mg/kg | 6 |

## 5.4	Groundwater Findings

### 5.4.1	Field Observations

Monitoring wells were gauged with an oil-water interface probe to evaluate the presence of non-aqueous phase liquid (NAPL) or other possible oil-related product; neither NAPL nor other oily substances were detected in the monitoring wells. Odors and/or sheen were not apparent at any monitoring wells during purging or sampling. Monitoring well PID headspace readings above background concentrations were not observed.

### 5.4.2	Analytical Results

Nine groundwater samples, plus one field duplicate, were collected and analyzed for Part 375/TCL VOCs, SVOCs, pesticides, herbicides, PCBs, Part 375/TAL total and dissolved TAL metals, 1,4-dioxane, and PFAS. Groundwater sample analytical results are summarized in Table 5 with comparisons to the SGVs. Groundwater sample locations and results exceeding the SGVs are presented on Figure 8, and the laboratory analytical reports are included in Appendix K.

The following contaminants were detected at concentrations exceeding SGVs. Concentrations are shown in micrograms per liter (µg/L).

VOCs

Three VOCs were detected above the SGVs in six groundwater samples from monitoring wells RIMW01, RIMW02, RIMW13, RIMW15, and RIMW17, as summarized in the following table:

| Analyte | Minimum Detected Concentration above SGVs | Maximum Detected Concentration above SGVs | SGVs | Frequency of Detection above SGV |
| --- | --- | --- | --- | --- |
| Chloroform | 20 µg/L in RIMW15_101725 | 20 µg/L in RIMW15_101725 | 7 µg/L | 1 |
| PCE | 5.1 µg/L in GWDUP01_101725 | 16 µg/L in RIMW17_101725 | 5 µg/L | 5 |
| TCE | 5.2 µg/L in RIMW15_101725 | 13 µg/L in RIMW01_101625 | 5 µg/L | 5 |

- Groundwater sample RIMW13_101725 is the parent sample of GWDUP01_101725.

SVOCs

One SVOC, benzo(a)anthracene, was detected above the SGV (0.002 µg/L) in one groundwater sample from monitoring well RIMW18 (0.04 µg/L).

Pesticides

Two pesticides were detected above the SGVs in three groundwater samples from monitoring wells RIMW05, RIMW10, and RIMW18, as summarized in the following table:

| Analyte | Minimum Detected Concentration above SGVs | Maximum Detected Concentration above SGVs | SGVs | Frequency of Detection above SGV |
| --- | --- | --- | --- | --- |
| Chlordane (alpha and gamma) | 0.682 µg/L in RIMW18_101725 | 1.32 µg/L in RIMW05_101625 | 0.05 µg/L | 2 |
| Heptachlor Epoxide | 0.038 µg/L in RIMW18_101725 | 0.063 µg/L in RIMW10_101725 | 0.03 µg/L | 3 |

PCBs and Herbicides

Total PCBs and herbicides were not detected at concentrations above the SGVs.

Total Metals

All groundwater samples contained concentrations of at least one total metal that exceeded the SGVs as summarized in the following table:

| Analyte | Minimum Detected Concentration above SGVs | Maximum Detected Concentration above SGVs | SGVs | Frequency of Detection above SGV |
| --- | --- | --- | --- | --- |
| Iron | 2,290 µg/L in RIMW01_101625 | 2,460 µg/L in RIMW05_101625 | 300 µg/L | 2 |
| Manganese | 491.4 µg/L in RIMW13_101725 | 2,430 µg/L in RIMW01_101625 | 300 µg/L | 4 |
| Sodium | 33,100 µg/L in RIMW05_101625 | 119,000 µg/L in RIMW01_101625 | 20,000 µg/L | 10 |

Dissolved Metals

All groundwater samples contained concentrations of at least one dissolved metal that exceeded the SGVs as summarized in the following table:

| Analyte | Minimum Detected Concentration above SGVs | Maximum Detected Concentration above SGVs | SGVs | Frequency of Detection above SGV |
| --- | --- | --- | --- | --- |
| Iron | 506 µg/L in RIMW18_101725 |  | 300 µg/L | 1 |
| Manganese | 489 µg/L in GWDUP01_101725 | 490.5 µg/L in RIMW13_101725 | 300 µg/L | 2 |
| Sodium | 28,800 µg/L in RIMW15_101725 | 120,000 µg/L in RIMW01_101625 | 20,000 µg/L | 10 |

PFAS

All groundwater samples contained PFOS and/or PFOA at concentrations that exceeded the SGVs as summarized in the following table:

| Analyte | Minimum Detected Concentration above SGVs | Maximum Detected Concentration above SGVs | SGVs | Frequency of Detection above SGV |  |
| --- | --- | --- | --- | --- | --- |
| Perfluorooctanesulfonic Acid (PFOS) | 0.00719 µg/L in RIMW02_101625 | 0.304 µg/L in RIMW13_101725 | 0.0027 µg/L | 10 |
| Perfluorooctanoic Acid (PFOA) | 0.00899 µg/L in RIMW02_101625 | 0.0366 µg/L in RIMW22_102025 | 0.0067 µg/L | 9 |

## 5.5	Soil Vapor Findings

Ten soil vapor samples and one outdoor ambient air sample were collected and submitted for laboratory analysis for USEPA TO-15 VOCs. Soil vapor sample results are summarized in Table 6. Soil vapor sample results are shown on Figure 9, and the laboratory analytical reports are included in Appendix K.

### 5.5.1	Analytical Results

Total VOCs in soil vapor samples ranged from 156.23 µg/m³ in RISV22 to 1,873.45 µg/m³ in RISV17. No standard exists for soil vapor samples in New York State. Total VOCs in the outdoor ambient air sample AA01 were detected at a concentration of 19.39 µg/m³. VOCs detected in soil vapor samples include the following:

| - 1,1,1-Trichloroethane - 1,2,4-Trimethylbenzene - 1,3,5-Trimethylbenzene - 2,2,4-Timehtylpentane - 2-Hexanone - 4-Ethyltoluene - Acetone - Benzene - Carbon Disulfide - Carbon Tetrachloride | - Chloroethane - Chloroform - Chloromethane - Cyclohexane - Dichlorodifluoromethane - Ethanol - Ethylbenzene - Isopropanol - M,P-Xylene - Methyl Ethyl Ketone - Methyl Isobutyl Ketone | - Naphthalene - n-Heptane - n-Hexane - o-Xylene - Tert-Butyl Alcohol - PCE - Tetrahydrofuran - Toluene - TCE - Trichlorofluoromethane |
| --- | --- | --- |

## 5.6	Quality Control Results

Duplicates, field blanks, PFAS field blanks, trip blanks, and MS/MSD samples were collected for quality control purposes and are summarized in Table 1. The duplicates and MS/MSD sample pairs for soil and groundwater were collected at a frequency of 1 per 20 primary samples (and one PFAS field blank per day of PFAS sampling). Quality control sample results were evaluated during data validation. Duplicate sample results are presented in Tables 3 through 5. Laboratory analytical reports are provided in Appendix K.

## 5.7	Data Usability

Category B laboratory reports for soil, groundwater, soil vapor, and ambient air samples collected during the RI were provided by Pace and submitted to Langan’s data validator. DUSRs are provided in Appendix J. The results of the data validation review are summarized below.

The data were determined to be acceptable. Completeness, defined as the percentage of analytical results that are judged to be valid, is 100 percent. No major deficiencies were identified. All data is considered useable as qualified.

## 5.8	Evaluation of Potential AOCs

This section discusses the findings of the RI with respect to potential AOCs identified in Section 3.4. The nature and extent of the site contamination are discussed separately in Section 7.0.

### 5.8.1	AOC 1: Suspected Historic Non-Native Backfilling

Non-native fill located throughout the site contains SVOCs, metals, pesticides, and PFAS at concentrations above the UU and/or RURR SCOs. The non-native fill layer extends from below the surface cover to depths ranging from about 1 to 5 feet bgs. The fill predominantly consists of tan to dark brown fine-grained sand with varying amounts of silt, clay, gravel, brick, concrete, glass, woody vegetation, and coal.  The source of SVOCs, metals, pesticides, and PFAS identified in site soil may be attributed to historical operations at the site, and use of contaminated fill to backfill the site after demolition of the former structures circa 1972 prior to the construction of the former motel on the site.

AOC 1 - Soil

Field screening and analytical results from non-native fill samples are summarized as follows:

Non-native fill was observed from below the surface cover down to about 5 feet bgs.

Six SVOCs (benzo[a]anthracene, benzo(a)pyrene, benzo(b)fluoranthene, chrysene, dibenz(a,h)anthracene, and indeno[1,2,3-cd]pyrene) were detected at concentrations above the applicable UU and/or RURR SCOs in six samples collected from 0 to 5 feet bgs throughout the site.

Four pesticides (4,4’-DDE, 4,4’-DDT, alpha chlordane, and heptachlor) were detected above UU SCOs in 16 soil samples collected from 0 to 6 feet bgs throughout the site.

Five metals (copper, lead, mercury, nickel, and zinc) were detected at concentrations above UU SCOs at depths between 0 and 5 feet bgs. Mercury (RISB15_0-2) and lead (RISB19_0-1 and RISB20_0-2) were also detected at a concentration above RURR SCOs in soil samples collected from shallow soil in the southern and western parts of the site.

PFOS was detected at a concentration above UU guidance value in eight soil samples collected from 0 to 5 feet bgs throughout the site.

AOC 1 – Groundwater

Of the compounds commonly associated with non-native fill (i.e., SVOCs and metals), one or more of three total metals (iron, manganese, and sodium) were detected above the SGVs in unfiltered groundwater in all monitoring wells throughout the site. Iron was detected above its SGV in the northern part of the site (RIMW01 and RIMW05), manganese was detected above its SGV in the northern and central parts of the site (RIMW01, RIMW05, and RIMW13), and sodium was detected in all unfiltered groundwater samples throughout the site. Filtered groundwater samples also contained three dissolved metals (iron, manganese, and sodium). PFOS and PFOA were also detected at concentrations above the SGVs across the site.

AOC 1 – Soil Vapor

Thirty-two VOCs were detected in soil vapor samples throughout the site. However, VOCs were not detected in any RI soil samples above the UU SCOs.

AOC 1 – Conclusions

Non-native fill throughout the site contains SVOCs, pesticides, and metals at concentrations above the UU and/or RURR SCOs. Concentrations of SVOCs, pesticides, and metals are attributed to releases from historical operations and/or the quality of backfill used during historical development of the site.

One SVOC, benzo(a)anthracene, and two pesticides, heptachlor and chlordane, were detected at concentrations exceeding the SGVs in groundwater samples collected from monitoring wells throughout the site; however, non-native fill, identified at depths ranging from 1 to 5 feet bgs at this site, is not in contact with groundwater (about 20 feet bgs), and does not appear to have leached into native soil since these compounds were not detected in native soil below the non-native fill. Benzo(a)anthracene, chlordane and heptachlor identified in groundwater are likely related to an off-site source or sources.

Metals were detected in groundwater at concentrations above the SGVs (iron, manganese, sodium) throughout the site; however, it is unlikely fill contributes to groundwater quality given the fill layer was observed at a maximum depth of 5 feet bgs while groundwater was observed at about 20 feet bgs. The dissolved metal detections in groundwater are likely associated with regional groundwater conditions.

PFOA and PFOS were identified in groundwater samples above the SGVs; however, an on-site source was not identified.

Historical operations do not appear to have impacted soil vapor. Petroleum-related and chlorinated VOCs identified in soil vapor are likely attributed to an off-site source.

### 5.8.2	AOC 2: CVOC Impacts in Groundwater and Soil Vapor

The Phase II ESA and SSI identified the presence of TCE and PCE in groundwater and soil vapor across the site. During the Phase II ESI, concentrations of TCE and/or PCE exceeded the SGVs in three groundwater monitoring wells across the site. Maximum TCE and PCE concentrations of 176 µg/m3 and 228 µg/m3 were detected during the SSI in soil vapor samples SV03 and SV01, respectively, which confirmed a VEC exists at the site.

AOC 2 – Soil

CVOCs were not detected in any soil samples above the UU SCOs. PCE and/or TCE were detected at concentrations below the UU SCOs in 12 soil samples at depths ranging from 0 to 6.5 feet bgs across the site. A majority of the detections were assigned “J” qualifiers, indicating that the analyte was positively identified and the associated numerical value is the approximate concentration of the analyte in the sample. Chloroform was not detected in any soil samples collected at the site.

AOC 2 – Groundwater

Three CVOCs (chloroform, PCE, and TCE) were detected in five monitoring wells above the SGVs. Chloroform was detected above its SGV in the central part of the site (RIMW15), PCE was detected above its SGV in the southern, central and southern parts of the site (RIMW01, RIMW02, RIMW13, and RIMW17), and TCE was above its SGV in the northern and central parts of the site.

AOC 2 – Soil Vapor

Eight CVOCs (1,1,1-trichloroethane, carbon tetrachloride, chloroethane, chloroform, chloromethane, dichlorodifluoromethane, PCE, and TCE) were detected in soil vapor samples throughout the site. PCE and TCE were detected in all soil vapor samples throughout the site. PCE concentrations ranged from 1.77 µg/m3 (RISV22) in the western part of the site to 1,630 µg/m3 (RISV17) in the southern part of the site. TCE concentrations ranged from 2.56 µg/m3 (RISV22) in the western part of the site to 245 µg/m3 (RISV13) in the central part of the site. Low levels of chloromethane and dichlorodifluoromethane were detected in the ambient air sample.

AOC 2 – Conclusions

Three CVOCs (chloroform, PCE, and TCE) were detected in five monitoring wells above the SGVs. Eight CVOCs (1,1,1-trichloroethane, carbon tetrachloride, chloroethane, chloroform, chloromethane, dichlorodifluoromethane, PCE, and TCE) were detected in soil vapor samples throughout the site. CVOCs were not detected above the UU SCOs in soil samples collected throughout the site. The detection of CVOCs may be associated with either a localized on-site impact or potential on-site migration from an off-site source.

### 5.8.3	AOC 3: Suspect UST in the Northwestern Part of the Site

During the SSI and RI, a geophysical anomaly was identified in the northwestern part of the site which indicated the potential presence of a UST. Additional borings (RISB24, RISB25, and RISB26) were advanced in the vicinity of the suspect UST to investigate the potential for petroleum-related impacts to the subsurface.

AOC 3- Soil

Field evidence (visual and/or olfactory) of petroleum impacts was not observed in the soil borings; and PID readings were not detected above background concentrations, indicating no detectable levels of VOCs were present. Concentrations of VOCs and SVOCs were not detected in soil samples collected from RISB24, RISB25, and RISB26. One additional soil boring was located in the vicinity of suspect UST (RISB18); soil samples collected from this boring did not exceed the UU SCOs for VOCs or SVOCs. Soil sample analytical results for soil borings RISB24, RISB25, and RISB26 are summarized on Table 3 and presented on Figure 7B.

AOC 3 – Groundwater

One monitoring well (RIMW18) was installed in the vicinity of the suspect UST; concentrations of VOCs were not detected above the SGVs in the associated groundwater sample. One SVOC, benzo(a)anthracene, was detected above the SGV in the groundwater sample collected from this well.

AOC 3 – Soil Vapor

One soil vapor point (RISV22) was installed to 5 feet bgs in the vicinity of the suspect UST; the concentration of total benzene, toluene, ethylbenzene, and xylenes (BTEX) was 95.88 µg/m³.

AOC 3 – Conclusions

Concentrations of VOCs and SVOCs were not detected in soil samples collected from soil borings RISB24, RISB25, and RISB26, and field observations did not indicate the presence of petroleum impacts. A groundwater sample collected in the vicinity of the suspect UST did not contain concentrations of VOCs above the SGVs. One SVOC, benzo(a)anthracene, was detected above the SGV, and is likely attributed to an off-site source. Petroleum-related VOCs were detected in a soil vapor sample collected in the vicinity of the suspect UST; however, the soil vapor sample concentrations are consistent with concentrations identified throughout the site. Based on the lack of impacts in soil and groundwater, this area is no longer considered an AOC.

### 5.8.4	AOC 4: Lead Hotspot in the Northwestern Part of the Site

During the Phase II ESA, lead was detected at a concentration of 3,000 mg/kg from 0 to 2 feet bgs in the northwestern part of the site; therefore, the area was delineated as part of the RI to determine the vertical and horizontal extents of elevated lead concentrations.

AOC 4 – Soil

Concentrations of lead exceeded the UU SCOs at the central boring location (RISB18) and at the five-foot step-outs to the northeast and southwest. Concentrations of lead in all soil samples from this delineation did not exceed the RURR SCOs. One sample, RISB18_NE1_0-2, was analyzed for TCLP lead and did not exceed the USEPA hazardous waste regulatory level of 5 mg/kg.

AOC 4 – Groundwater

Lead was not detected above the SGV in monitoring well RIMW18.

AOC 4 – Conclusions

Lead was not detected above the RURR SCOs in the central boring and delineation soil samples. The maximum lead concentration above the UU SCO was 328 mg/kg (RISB18_NE1_0-2); the associated TCLP lead sample concentration did not exceed the hazardous waste regulatory level of 5 mg/kg and was therefore not considered hazardous. Additionally, lead was not identified above the SGV in groundwater collected from this location. Although lead was detected at a concentration of 3,000 mg/kg from 0 to 2 feet bgs during the Phase II ESI, the RI did not identify evidence of further lead impacts. Therefore, this area has been fully delineated.

### 5.8.5	AOC 5: Lead Hotspot in the Western Part of the Site

During the SSI, lead was detected at a concentration of 1,040 mg/kg from 1 to 2 feet bgs in the western part of the site; therefore, the area was delineated as part of the RI to determine the vertical and horizontal extents of elevated lead concentrations.

AOC 5 – Soil

Concentrations of lead exceeded the UU SCOs at the central boring location (RISB21) and at the five-foot step-outs to the north and west. Concentrations of lead in all soil samples from this delineation did not exceed the RURR SCOs. One sample, RISB21_N1_0-2, was analyzed for TCLP lead and did not exceed the USEPA hazardous waste regulatory level of 5 mg/kg.

AOC 5 – Groundwater

Lead was not detected above the SGV in monitoring well RIMW18.

AOC 5 – Conclusions

Lead was not detected above the RURR SCOs in the central boring and delineation soil samples. The maximum lead concentration above the UU SCO was 326 mg/kg (RISB21_N1_0-2); the associated TCLP lead sample concentration did not exceed the hazardous waste regulatory level of 5 mg/kg and was therefore not considered hazardous. Additionally, lead was not identified above the SGV in groundwater collected from this location. Although lead was detected at a concentration of 1,040 mg/kg from 1 to 2 feet bgs during the SSI, the RI did not identify evidence of further lead impacts. Therefore, this area has been fully delineated.

# GREEN REMEDIATION STANDARDS

Green remediation principles and techniques were implemented to the extent feasible in the design and implementation of the RI as per DER-31. DER-31 requires the following green remediation/sustainability concepts be considered and/or implemented, to the extent feasible, during investigations:

- Increase energy efficiency/minimize total energy use and direct and indirect carbon dioxide/greenhouse gas (GHG) emissions to the atmosphere;

- Reduce emissions of air pollutants;

- Minimize habitat disturbance and create or enhance habitat or usable land;

- Conserve natural resources such as soil and water; promote the sequestration of carbon through reforestation or afforestation;

- Minimize freshwater consumption and maximize water reuse during daily operations and treatment processes;

- Prevent long-term erosion, surface runoff, and off-site water quality impacts, and prevent unintended soil compaction;

- Minimize waste or implement beneficial use of materials that would otherwise be considered a waste;

- Minimize equipment and truck idling and use sustainably produced biofuels to reduce discharges of pollutants and GHGs to the atmosphere;

- Utilize clean diesel (new or retrofitted) equipment to reduce emissions to the atmosphere;

- Minimize truck travel for disposal to save energy, reduce emissions, reduce localized noise, vibration, and wear and tear on roads; and

- Minimize use of heavy equipment to save energy and reduce emissions.

During the RI, the following elements were implemented, to the extent feasible, to reduce GHG and other emissions:

- Use of Ultra Low Sulfur Diesel in vehicles and machinery by drilling contractor;

- Use of diesel exhaust purifier scrubbers on machinery (drill rigs) by drilling contractor;

- Minimization of vehicle idling of all vehicles (including construction equipment) in accordance with 6 NYCRR Part 217 Motor Vehicle Emissions, Subpart 217-3 Idling Prohibition for Heavy Duty Vehicles; and

- Reduction of materials consumption and off-site transport by reuse of non-impacted soil cuttings within the boring of origin and containerizing of soil cuttings exhibiting visual, olfactory, and instrumental signs of contamination.

# QUALITATIVE HUMAN HEALTH EXPOSURE ASSESSMENT

Human health exposure risk was evaluated for both current and future site and off-site conditions, in accordance with DER-10. The assessment includes an evaluation of potential sources and migration pathways of site contamination, potential receptors, exposure media, and receptor intake routes and exposure pathways.

In addition to the human health exposure assessment, NYSDEC DER-10 requires an on-site and off-site Fish and Wildlife Resources Impact Analysis (FWRIA) if certain criteria are met. According to the requirements stipulated in Section 3.10 and Appendix 3C of DER-10, an FWRIA is not required for the site. A completed form of DER-10 Appendix 3C is included in Appendix L.

## 7.1	Current Conditions

The about 33,700-square-foot (±0.77-acre) site is located at 615 Boadway in the city of Kingston, New York and is identified on the Ulster County Tax Map as Section 56.109, Tax Block 3, Lot 16.  The site is currently vacant and improved with an asphalt-paved parking lot and a landscaped area in the eastern part of the site fronting Broadway.

## 7.2	Proposed Conditions

The proposed development a three- to five-story mixed-use building comprised of two towers which contain residential and commercial units and community space. The ground floor of Tower 1 will contain a lobby, commercial tenant spaces, a bike room, and a mechanical room/office space; the ground floor of Tower 2 will contain an open-air parking lot. The remaining floors will contain residential units. The new building will cover about 21,800 square feet of the site footprint. Areas outside of the building footprint will be developed into a parking lot, paved walkways, landscaped areas, and a plaza.

## 7.3	Summary of Environmental Conditions

AOCs include historic on-site operations and/or quality of backfill used at the site and CVOC impacts to groundwater and soil vapor. The following contaminants of concern (COC) were identified:

SVOCs, pesticides and metals in soil

CVOCs in groundwater and soil vapor

Soil Impacted with SVOCs, Pesticides, and Metals

SVOCs, pesticides and metals were detected at concentrations above the UU, PGW and/or RURR SCOs in samples collected from non-native fill across the site. Generally, SVOCs, pesticides and metals were detected at concentrations that are typical of non-native fill material. SVOC, pesticide and metal detections are attributed to historic on-site operations and/or the quality of backfill used at the site.

CVOC-Impacted Groundwater and Soil Vapor

Three VOCs (chloroform, PCE, and TCE) were detected above the SGVs in five groundwater samples in the northern, central, and southern parts of the site. Eight CVOCs (1,1,1-trichloroethane, carbon tetrachloride, chloroethane, chloroform, chloromethane, dichlorodifluoromethane, PCE, and TCE) were detected in soil vapor samples throughout the site. CVOCs were not detected above the UU SCOs in soil samples collected throughout the site. The detection of CVOCs in soil vapor and groundwater is attributed to either a localized on-site impact or potential on-site migration from an off-site source.

## 7.4	Conceptual Site Model

A conceptual site model (CSM) has been developed based on the findings of the RI. The purpose of the conceptual site model is to develop a simplified framework for understanding the distribution of impacted materials, potential migration pathways, and potentially complete exposure pathways, as discussed below.

### 7.4.1	Potential Sources of Contamination

Potential sources of contamination have been identified as historical site uses and backfilling activities, and unknown off-site sources.

Historical development and backfilling activities and historical use of the site may be a source of SVOCs, pesticides, and metals that were detected above the UU, PGW and/or RURR SCOs across the site.

PFOS was detected in soil samples above the UU guidance value, and PFOA and PFOS were identified in groundwater samples above the SGVs across the site; however, an on-site source was not identified.

CVOCs were detected in groundwater and soil vapor samples throughout the site and are attributed to either a localized on-site impact or potential on-site migration from an off-site source.

### 7.4.2	Exposure Media

The impacted media at the site include soil, groundwater, and soil vapor. SVOCs, pesticides, and metals were detected above the Part 375 UU, PGW and/or RURR SCOs in soil samples throughout the site and are present at concentrations typical of historic fill. PFOS was detected in soil above the UU guidance values, and PFOS and PFOA were identified in groundwater above the SGVs; however, an on-site source was not identified.

Groundwater at the site contains one SVOC, pesticides, and metals at concentrations above the SGVs. The pesticides and SVOC are likely related to an off-site source or sources. Metal detections in groundwater may be attributable to regional conditions.

Groundwater samples collected from monitoring wells across the site contained CVOCs above the SGVs. CVOCs were also detected in soil vapor throughout the site at concentrations that could potentially warrant mitigation in the absence of remediation.

### 7.4.3	Receptor Populations

The site is currently vacant and improved with an asphalt-paved parking lot and a landscaped area in the eastern part of the site fronting Broadway. Human receptors under the current conditions include site workers/visitors and personnel performing routine maintenance and environmental sampling. During site development, human receptors will be limited to construction and remediation workers and authorized guests visiting the site. Under future conditions, receptors will include the residential use occupants, employees, and the nearby community, including children.

## 7.5	Potential Exposure Pathways – On-Site

### 7.5.1	Current Conditions

Human exposure to contaminated soil is prevented by an impermeable asphalt cover and vegetated areas that overly the site. Site access is partially restricted by a metal fence; therefore, human exposure to contaminated soil is limited. There is a potential pathway through dermal absorption, inhalation and ingestion for investigation workers that handle soil beneath the surface cover materials, but this is controlled by implementation of a site-specific Health and Safety Plan (HASP).

Groundwater in the urban areas of Kingston is not used as a potable water source. There is a potential exposure pathway during groundwater sampling associated with site investigation. The potential pathway for site investigation workers is through dermal absorption and ingestion but is controlled by implementation of a site-specific HASP.

Soil vapor contains VOCs at concentrations that may warrant vapor mitigation per the NYSDOH Decision Matrices. However, there are no permanent, enclosed structures on-site where vapors may potentially accumulate.

### 7.5.2	Construction/Remediation Conditions

Construction and remediation may result in potential exposures to site contaminants in the absence of a HASP. Construction and remediation activities will likely include excavation and off-site disposal of contaminated fill and soil, potential dewatering, potential construction of a composite cover system, potential installation of a vapor mitigation system, and construction of foundation components. In the absence of a HASP, this scenario presents the potential for exposure of construction and remediation workers to soil and groundwater, and potential soil vapor contaminants via dermal absorption, ingestion, and inhalation of vapors and particulate matter. These exposure pathways to soil, groundwater, and soil vapor will be minimized through the implementation of the HASP, which will include on-site air monitoring and requirements for vapor and dust suppression techniques.

### 7.5.3	Proposed Future Conditions

The proposed development includes a three- to five-story, mixed-use, slab-on-grade building. The site is anticipated to undergo a Track 2 cleanup.

There is no complete exposure pathway for ingesting residual groundwater impacts, because the site and surrounding areas within the urban areas of Kingston obtain their drinking water supply from surface water reservoirs and not from groundwater. The potential pathway for soil vapor intrusion into the future on-site building would be mitigated by installation of a sub-membrane depressurization system.

If necessary, institutional controls will require maintenance of engineering controls and will serve to further mitigate exposure under future conditions.

## 7.6	Potential Exposure Pathways – Off-Site

The potential off-site migration of site soil, groundwater, and/or soil vapor contaminants is not expected to result in a complete exposure pathway for current, construction and remediation, or future conditions for the following reasons:

The site is located in an urban area and predominantly covered with continuous impervious surface covering.

During site remediation and construction, the following protective measures and/or conditions will be in place:

  - A site-specific HASP, including a Community Air Monitoring Plan (CAMP), will be implemented to protect on-site personnel and to monitor the perimeter of the site to mitigate off-site migration of particulates and VOCs during construction.

  - Air monitoring will be conducted for particulates (i.e., dust) and VOCs during ground-intrusive activities as part of a CAMP. Dust and/or vapor suppression techniques will be employed to limit the potential for off-site migration of dust and vapors.

  - Vehicle tires and undercarriages will be washed as necessary prior to leaving the site to prevent tracking soil off-site.

  - If dewatering of groundwater is required to accommodate excavation, dewatering fluids may be pre-treated and discharged to a maintained stormwater sewer under a NYSDEC State Pollution Discharge Elimination System (SPDES) permit or containerized in a temporary storage tank pending disposal at a permitted off-site facility.

A soil erosion/sediment control plan will be implemented during construction to control off-site migration of soil.

There is no pathway for ingesting residual groundwater impacts, as the site and surrounding areas obtain their drinking water supply from surface water reservoirs located upstate and not from groundwater.

## 7.7	Evaluation of Human Health Exposure

Based on the CSM and review of environmental data, complete on-site exposure pathways appear to be present, in the absence of monitoring and mitigation or engineering controls in construction/remediation and future use conditions.

Complete exposure pathways have the following five elements: 1) a contaminant source; 2) a contaminant release and transport mechanism; 3) a point of exposure; 4) a route of exposure; and 5) a receptor population. A discussion of the five elements comprising a complete pathway as they pertain to the site is provided below.

### 7.7.1	Current Conditions

Contaminant sources include varying concentrations of SVOCs, pesticides, metals, and PFAS in soil and/or groundwater and VOCs in groundwater and soil vapor.

Contaminant release and transport mechanisms include contaminated soil transported as dust (dermal, ingestion, inhalation), and exposure to potential soil vapor contaminants (inhalation). Under current conditions, the likelihood of human exposure is limited, as the groundwater beneath the site is not a source of drinking water.

### 7.7.2	Construction/Remediation Activities

During development and remediation, the contaminant sources are the same as under current conditions. Points of exposure include disturbed and exposed soil during excavation, dust and organic vapors generated during excavation, and contaminated groundwater that may be encountered during excavation and dewatering. Routes of exposure include ingestion and dermal absorption of contaminated soil and groundwater and inhalation of dust and vapor emanating from contaminated soil. The receptor population includes construction and remediation workers and, to a lesser extent, workers and visitors adjacent to the site.

The potential for completed exposure pathways is present since all five elements exist; however, the risk can be avoided or minimized by applying appropriate health and safety measures during construction and remediation, such as monitoring the air for organic vapors and dust, using vapor and dust suppression measures, cleaning truck undercarriages and securing tarp covers before trucks leave the site to prevent off-site soil tracking, maintaining site security, and wearing the appropriate personal protective equipment (PPE).

A HASP, a RAWP, and a CAMP that include measures such as conducting a community air-monitoring program, donning PPE, covering soil stockpiles, altering work sequencing, restricting eating and drinking on-site, maintaining a secure construction entrance, proper housekeeping, and applying vapor and dust suppression measures to prevent off-site migration of vapors and particulates during construction will be implemented. Such measures will prevent completion of exposure pathways for soil, groundwater, and soil vapor contaminants.

### 7.7.3	Proposed Future Conditions

For the proposed future conditions, a Track 2 cleanup is anticipated. Residual contaminants may remain on-site following completion of the forthcoming Remedial Action and would include those listed under current conditions. If institutional and/or engineering controls are not implemented, points of exposure may include potential cracks in the foundation or slab of the proposed development, and exposure during any future soil-disturbing activities. The receptor population includes potential building tenants and/or employees, visitors, and maintenance workers. The possible routes of exposure can be avoided or mitigated by the installation of engineering controls, such as a vapor mitigation system, and the implementation of institutional controls, such as restrictions on groundwater and land use and execution of a Site Management Plan (SMP), if necessary.

### 7.7.4	Human Health Exposure Assessment Conclusions

Under current conditions, there is a marginal risk for exposure. The primary exposure pathways are dermal contact, ingestion and inhalation of soil, dust, or groundwater by authorized site visitors in instances where the impermeable site cover is compromised or during site investigation. The exposure risks can be avoided or minimized by following the appropriate HASP and vapor and dust suppression measures, and by implementing a CAMP during intrusive activities.

In the absence of engineering controls, there is a potential for exposure during the construction and remediation activities. The primary exposure pathways are:

Dermal contact, ingestion and inhalation of contaminated soil, groundwater or soil vapor by construction workers.

Dermal contact, ingestion and inhalation of soil (dust) by the community in the vicinity of the site.

These can be avoided or minimized by performing community air monitoring and by following the appropriate health and safety, dust suppression, and site security measures outlined in a site-specific HASP.

The existence of a complete exposure pathway for site contaminants to human receptors during the proposed future use condition is unlikely, as most contaminated soil will be excavated and transported to an off-site disposal facility. Regional groundwater is not used as a potable water source. The potential pathway for soil vapor intrusion into the future on-site building would be addressed by installation of a waterproof vapor barrier as an element of green construction and a sub-membrane depressurization system as an engineering control, which would minimize the risk of soil vapor intrusion.

It is possible that a complete exposure pathway exists for the migration of site contaminants to off-site human receptors during current, construction-phase, and future conditions. Monitoring and control measures have been and will continue to be used during remediation/construction to prevent completion of this pathway. Under future conditions, the site will be remediated, and engineering and institutional controls will be implemented, if necessary, to prevent completion of this pathway.

# NATURE AND EXTENT OF CONTAMINATION

This section evaluates the nature and extent of soil, groundwater, and soil vapor contamination. The nature and extent of the contamination is derived from a combination of field observations and analytical data that were discussed in Section 5.0. The nature and extent of contamination also includes results from sampling data obtained prior to execution of the RIWP.

## 8.1	Soil Contamination

Soil containing non-native fill extends to about 5 feet bgs throughout the site. During the RI, several SVOCs, pesticides, and metals were detected at concentrations above the UU, PGW and/or RURR SCOs in the fill layer. PFOS was detected at a concentration above UU guidance values from about 0 to 5 feet bgs. The deepest observed concentrations above the UU SCOs were at about 6 feet bgs, and about 5 feet bgs for RURR SCOs.

## 8.2	Groundwater Contamination

VOCs (specifically CVOCs), SVOCs, pesticides, total and dissolved metals, and PFOS/PFOA were detected above SGVs in groundwater throughout the site. Based on the depths between the fill layer and the groundwater table, concentrations of contaminants in soil do not appear to be affecting groundwater quality; therefore, the presence of SVOCs, pesticides and PFOS/PFOA is attributed to off-site sources. The concentrations of metals detected in groundwater are likely associated with regional groundwater conditions. The detection of CVOCs in groundwater may be associated with either a localized on-site impact or potential on-site migration from an off-site source.

## 8.3	Soil Vapor Contamination

VOCs were detected in soil vapor at concentrations that could potentially warrant mitigation in the absence of remediation per the NYSDOH Decision Matrices. VOCs were at their highest concentrations in the southern part of the site (RISV17). CVOCs (TCE and PCE) were identified in soil samples collected throughout the site but were detected below the UU SCOs. The source for soil vapor contamination may be associated with either a localized on-site impact or potential on-site migration from an off-site source.

# CONCLUSIONS

The RI was implemented between October 6 and November 26, 2025. The findings summarized herein are based on both qualitative and quantitative data, consisting of field observations, instrumental readings, and laboratory analytical results of soil, groundwater, and soil vapor samples collected during the RI.

Significant RI findings are summarized as follows:

- Stratigraphy:  The site cover is underlain by non-native fill, predominantly consisting of tan to tannish brown to dark brown fine-grained sand with varying amounts of silt, clay, gravel, brick, concrete, glass, woody vegetation, and coal, and extending from surface grade to about 1 to 5 feet bgs. Native soil that underlies the fill consists of tan to orangish brown fine-grained sand with varying amounts of clay, silt, and gravel. Bedrock was not encountered during the RI.

Hydrogeology:  Synoptic groundwater level measurements were collected from permanent monitoring wells on October 22, 2025. about 19.53 to 20.25 feet below top of casing, with groundwater elevations ranging from about el 163.73 to el 162.51. Groundwater elevation is highest in the northeastern part of the site, and groundwater flow at the site generally flows to the southwest.

Soil/Fill: SVOCs were detected at concentrations above the applicable UU and/or RURR SCOs in six soil samples collected from 0 to 5 feet bgs throughout the site. Pesticides were detected above UU SCOs in 16 soil samples collected from 0 to 6 feet bgs throughout the site. Metals were detected at concentrations above the applicable UU SCOs at depths between 0 and 5 feet bgs; mercury and lead were also detected at concentrations above RURR SCOs in soil samples collected from shallow soil in the southern and western parts of the site. PFOS was detected at a concentration above the UU guidance value in eight soil samples collected from 0 to 5 feet bgs across the site. The source of SVOCs, pesticides and metals in soil are likely attributed to releases from historical operations and/or the quality of backfill used during historical development of the site. The presence of PFOS is attributed to an unknown off-site source, as no on-site sources or historical uses were identified.

Groundwater: CVOCs including chloroform, PCE, and TCE were detected in five monitoring wells above the SGVs. Total and dissolved metals were detected above their SGVs throughout the site. PFOS and PFOA were identified throughout the site at concentrations above the NYSDEC SGVs. The source of CVOCs in groundwater may be associated with either a localized on-site impact or potential on-site migration from an off-site source. The dissolved metals detections in groundwater are likely associated with regional groundwater conditions. An on-site source of PFOA and PFOS was not identified. One SVOC, benzo(a)anthracene, and two pesticides, heptachlor and chlordane, were detected at concentrations exceeding the SGVs in groundwater samples collected monitoring wells throughout the site; however, based on depths between the fill layer and the groundwater table, soil detections do not appear to be affecting groundwater quality and are likely related to an off-site source or sources.

Soil Vapor: CVOCs including PCE and TCE were detected in soil vapor at concentrations that may warrant mitigation. The source of CVOCs in soil vapor may be attributed to either a localized on-site impact or potential on-site migration from an off-site source.

Sufficient analytical data were gathered during the RI to establish site-specific soil cleanup levels and to develop a remedy for the site. The remedy will be described and evaluated in the forthcoming RAWP which will be prepared in accordance with New York State BCP guidelines. The remedy will address impacts to soil, groundwater, and soil vapor described in this RIR.

# REFERENCES

- Epstein, Jack B., Lyttle, Peter T, Preliminary Geologic Map and Cross Section of the Ellenville and Kerhonkson Quadrangles, 1990

- Epstein, Jack B., Lyttle, Peter T, Preliminary Geologic Map and Cross Section of the Napanoch Quadrangle, Ulster and Orange Counties, New York, 1990

- Weston & Sampson, Phase I Environmental Site Assessment Report, March 2019.

- Weston & Sampson, Phase II Environmental Site Assessment Report, July 2020.

- Langan Engineering, Environmental, Surveying, and Landscape Architecture, D.P.C., Supplemental Subsurface Investigation September 30, 2024.

- Langan Engineering, Environmental, Surveying, and Landscape Architecture, D.P.C., Remedial Investigation Work Plan, May 13, 2025.

- New York State Division of Water Technical and Operational Guidance Series (TOGS) (1.1.1) dated June 1998

- Draft Brownfield Cleanup Program Guide, New York State Department of Environmental Conservation (NYSDEC), Division of Environmental Remediation, dated May 2004

- Final Guidance for the Evaluation of Soil Vapor Intrusion in the State of New York, New York State Department of Health, dated October 2006, revised 2024

- Part 375 of Title 6 of the New York Compilation of Codes, Rules, and Regulations, NYSDEC, effective December 14, 2006

- DER-10 Technical Guidance for Site Investigation and Remediation, NYSDEC, issued May 3, 2010; effective June 18, 2010

- Sampling, Analysis, and Assessment of Per- and Polyfluoroalklyl Substances (PFAS) under NYSDEC’s Part 375 Remedial Programs, NYSDEC, dated April 2023

- Low Flow Purging and Sampling Procedure for the Collection of Groundwater Samples from Monitoring Wells, United States Environmental Protection Agency, EQASOP-GW 001, January 19, 2010

- National Wetlands Inventory, United States Fish and Wildlife Service, https://www.fws.gov/wetlands/data/Mapper.htm

Elevations in this RIR are referenced to the North American Vertical Datum of 1988 (NAVD88), which is about 1.1 feet above mean sea level at Sandy Hook, New Jersey.